# streamlit_app.py — full script

# app.py

# Streamlit app: Asphalt NCR Checks (PDF + digital tables + header row finder + updated logic)

#

# Updated logic per Shannon's note:

# - Underlying layer cannot be higher than +U (max tolerance high). Therefore:

# * HIGH & THIN remains PHYSICALLY IMPOSSIBLE (would require underlying too high to "steal" thickness)

# * A "hard minimum feasible thickness" is pointwise:

# Tmin_point = D + LevelConformance - U

# If Thickness < Tmin_point => PHYSICALLY IMPOSSIBLE (CHECK SURVEY ERROR)

#

# - Underlying layer CAN be low (and may have an accepted NCR). Therefore:

# * LOW & THICK is NOT automatically a survey error.

# It is flagged as:

# "MAY BE ACCEPTABLE – underlying low NCR (thickness made up in asphalt)"

#

# - We also provide an informational flag:

# Thick > (D + LevelConformance + U)

# This means "thicker than would be possible IF underlying were within ±U".

# Not an impossibility, because underlying might be low beyond -U (accepted NCR).

#

# Install:

# pip install streamlit pandas numpy openpyxl camelot-py[cv]

#

# Run:

# streamlit run app.py

import os

import re

import tempfile

from typing import Optional, Tuple, List, Dict

import numpy as np

import pandas as pd

import streamlit as st

st.set_page_config(page_title="Asphalt NCR Checks", layout="wide")

st.title("Asphalt NCR Checks")

st.caption(

"Separates genuine construction nonconformances from physically impossible results "

"using first-principles geometry (level vs thickness sanity checks)."

)

# ---------------------------

# Helpers: general

# ---------------------------

def normalize_cols(df: pd.DataFrame) -> pd.DataFrame:

df = df.copy()

df.columns = [re.sub(r"\s+", " ", str(c).strip()) for c in df.columns]

return df

def find_col(df: pd.DataFrame, candidates: List[str]) -> Optional[str]:

cols_lc = {c.lower(): c for c in df.columns}

for cand in candidates:

if cand.lower() in cols_lc:

return cols_lc[cand.lower()]

for c in df.columns:

lc = c.lower()

for cand in candidates:

if cand.lower() in lc:

return c

return None

def to_numeric(series: pd.Series) -> pd.Series:

s = series.astype(str).str.replace(",", "", regex=False)

s = s.str.replace(r"[^\d\.\-\+]", "", regex=True)

return pd.to_numeric(s, errors="coerce")

def load_csv_excel(upload) -> pd.DataFrame:

name = upload.name.lower()

if name.endswith(".csv"):

return pd.read_csv(upload)

if name.endswith(".xlsx") or name.endswith(".xls"):

return pd.read_excel(upload)

if name.endswith(".txt"):

return pd.read_csv(upload, sep=None, engine="python")

raise ValueError("Unsupported file type.")

# ---------------------------

# Helpers: PDF + header finding

# ---------------------------

HEADER_KEYWORDS = [

"chainage", "ch", "offset", "off", "level", "conformance", "thickness", "ac", "rl", "mm"

]

def _cell_text(x) -> str:

if x is None:

return ""

s = str(x)

s = s.replace("\u00a0", " ")

s = re.sub(r"\s+", " ", s).strip()

return s

def _row_text(row: pd.Series) -> str:

return " ".join(_cell_text(v) for v in row.values if _cell_text(v))

def _keyword_hits(text: str) -> int:

t = text.lower()

return sum(1 for kw in HEADER_KEYWORDS if kw in t)

def _alpha_ratio(text: str) -> float:

if not text:

return 0.0

letters = sum(1 for c in text if c.isalpha())

return letters / max(len(text), 1)

def _numeric_ratio(text: str) -> float:

if not text:

return 0.0

nums = sum(1 for c in text if c.isdigit())

return nums / max(len(text), 1)

def detect_header_row_index(df: pd.DataFrame, scan_rows: int = 25) -> Optional[int]:

if df.empty:

return None

n = min(scan_rows, len(df))

best_i = None

best_score = -1.0

for i in range(n):

text = _row_text(df.iloc[i])

if not text:

continue

hits = _keyword_hits(text)

a = _alpha_ratio(text)

num = _numeric_ratio(text)

score = hits * 10.0 + a * 2.0 - num * 2.0

if hits == 0 and num > 0.35 and a < 0.15:

score -= 3.0

if score > best_score:

best_score = score

best_i = i

if best_i is None:

return None

best_text = _row_text(df.iloc[best_i])

if _keyword_hits(best_text) == 0:

return None

return best_i

def apply_header_row(df: pd.DataFrame, header_idx: int) -> pd.DataFrame:

df = df.copy()

header = [_cell_text(v) for v in df.iloc[header_idx].values]

header_clean = []

for j, h in enumerate(header):

h2 = re.sub(r"\s+", " ", h).strip()

if not h2:

h2 = f"col_{j}"

header_clean.append(h2)

seen: Dict[str, int] = {}

unique = []

for h in header_clean:

if h not in seen:

seen[h] = 1

unique.append(h)

else:

seen[h] += 1

unique.append(f"{h}_{seen[h]}")

out = df.iloc[header_idx + 1 :].copy()

out.columns = unique

out = out.reset_index(drop=True)

return out

def _parse_pages(pages: str, num_pages: Optional[int]) -> List[int]:

if not pages or pages.strip().lower() == "all":

return list(range(1, num_pages + 1 if num_pages else 1))

parts = [p.strip() for p in pages.split(",") if p.strip()]

out: List[int] = []

for p in parts:

if "-" in p:

a, b = p.split("-", 1)

try:

a_i = int(a)

b_i = int(b)

except Exception:

continue

out.extend(list(range(a_i, b_i + 1)))

else:

try:

out.append(int(p))

except Exception:

continue

return sorted(set(out))

def extract_tables_from_pdf(

pdf_bytes: bytes,

pages: str = "all",

flavor: str = "stream",

strip_text: str = "\n",

header_finder: bool = True,

header_scan_rows: int = 25,

) -> pd.DataFrame:

"""

Try camelot first; if unavailable, fall back to pdfplumber.

If both are missing, raise a RuntimeError with install instructions.

"""

# Try Camelot (preferred for digital table extraction)

try:

import camelot  # type: ignore

with tempfile.NamedTemporaryFile(delete=False, suffix=".pdf") as tmp:

tmp.write(pdf_bytes)

pdf_path = tmp.name

try:

tables = camelot.read_pdf(pdf_path, pages=pages, flavor=flavor, strip_text=strip_text)

if tables.n == 0:

raise RuntimeError(

"No tables were detected by Camelot. Try switching flavor stream/lattice or adjusting pages."

)

cleaned = []

for t in tables:

df = t.df.copy()

df.columns = [str(c) for c in df.columns]

if header_finder:

hidx = detect_header_row_index(df, scan_rows=header_scan_rows)

if hidx is not None:

df = apply_header_row(df, hidx)

cleaned.append(df)

out = pd.concat(cleaned, ignore_index=True)

out.columns = [re.sub(r"\s+", " ", str(c).strip()) for c in out.columns]

return out

finally:

try:

os.remove(pdf_path)

except Exception:

pass

except Exception:

# Camelot not available or failed — try pdfplumber as a graceful fallback

try:

import pdfplumber  # type: ignore

with tempfile.NamedTemporaryFile(delete=False, suffix=".pdf") as tmp:

tmp.write(pdf_bytes)

pdf_path = tmp.name

cleaned: List[pd.DataFrame] = []

with pdfplumber.open(pdf_path) as doc:

page_idxs = _parse_pages(pages, len(doc.pages))

for pi in page_idxs:

if pi - 1 < 0 or pi - 1 >= len(doc.pages):

continue

page = doc.pages[pi - 1]

tables = page.extract_tables()

for t in tables:

try:

df = pd.DataFrame(t)

except Exception:

continue

df = df.astype(object).where(pd.notnull(df), None)

df.columns = [str(c) for c in df.columns]

if header_finder:

hidx = detect_header_row_index(df, scan_rows=header_scan_rows)

if hidx is not None:

df = apply_header_row(df, hidx)

cleaned.append(df)

try:

os.remove(pdf_path)

except Exception:

pass

if not cleaned:

raise RuntimeError(

"No tables were detected by pdfplumber. Try other pages or provide a CSV/Excel file."

)

out = pd.concat(cleaned, ignore_index=True)

out.columns = [re.sub(r"\s+", " ", str(c).strip()) for c in out.columns]

return out

except Exception:

raise RuntimeError(

"Camelot is required for PDF extraction (recommended). Alternatively install pdfplumber.\n"

"Install with: pip install camelot-py[cv]  OR  pip install pdfplumber"

)

# ---------------------------

# Core logic

# ---------------------------

def compute_thickness(

df: pd.DataFrame,

thickness_col: Optional[str],

top_rl_col: Optional[str],

bot_rl_col: Optional[str],

) -> Tuple[pd.DataFrame, str]:

df = df.copy()

note = ""

if thickness_col:

vals = to_numeric(df[thickness_col])

med = float(np.nanmedian(vals.dropna())) if vals.dropna().size else np.nan

if np.isfinite(med) and med > 0 and med < 20:

vals_mm = vals * 1000.0

note = f"Using provided thickness column: '{thickness_col}' (detected values ~{med:.3g} → converted m→mm)"

df["Thickness_mm"] = vals_mm

else:

df["Thickness_mm"] = vals

note = f"Using provided thickness column: '{thickness_col}'"

return df, note

if top_rl_col and bot_rl_col:

top = to_numeric(df[top_rl_col])

bot = to_numeric(df[bot_rl_col])

th = top - bot

med = float(np.nanmedian(th.dropna())) if th.dropna().size else np.nan

if np.isfinite(med) and med > 0 and med < 20:

th_mm = th * 1000.0

note = f"Computed thickness = '{top_rl_col}' - '{bot_rl_col}' (detected units m → converted to mm)"

df["Thickness_mm"] = th_mm

else:

df["Thickness_mm"] = th

note = f"Computed thickness = '{top_rl_col}' - '{bot_rl_col}'"

return df, note

raise ValueError(

"Thickness cannot be derived. Provide either a Thickness column or both Top RL and Underlying RL columns."

)

def classify_points_updated(

df: pd.DataFrame,

D: float,

A_upper: float,

A_lower: float,

U_upper: float,

U_lower: float,

level_col: str,

) -> Tuple[pd.DataFrame, str]:

"""

Updated classification:

- Level nonconforming: LC > +A OR LC < -A

- PHYSICALLY IMPOSSIBLE if:

Thickness < (D + LC - U) [underlying cannot be higher than +U]

OR (LC >= +A and Thickness < D) [HIGH & THIN]

- LOW & THICK (LC <= -A and Thickness > D):

MAY BE ACCEPTABLE – underlying low NCR (thickness made up in asphalt)

- Otherwise:

GENUINE NONCONFORMANCE – physically possible

- Informational:

Thickness > (D + LC + U) implies thicker than possible IF underlying within ±U

(not impossibility because underlying may be low beyond -U).

"""

df = df.copy()

df["LevelConformance_mm"] = to_numeric(df[level_col])

# Heuristic: if level conformance values look like metres (small magnitudes), convert to mm

level_note = "LevelConformance assumed in mm"

try:

med_level = float(np.nanmedian(np.abs(df["LevelConformance_mm"].dropna()))) if df["LevelConformance_mm"].dropna().size else np.nan

if np.isfinite(med_level) and med_level > 0 and med_level < 20:

df["LevelConformance_mm"] = df["LevelConformance_mm"] * 1000.0

level_note = "Detected LevelConformance in metres → converted to mm"

except Exception:

pass

df["Thickness_mm"] = to_numeric(df["Thickness_mm"])

# Filter: level nonconforming

df["Is_Level_Nonconforming"] = (df["LevelConformance_mm"] > A_upper) | (

df["LevelConformance_mm"] < -A_lower

)

# Thickness diffs vs design

df["Thickness_Diff_vs_Design_mm"] = df["Thickness_mm"] - D

# Pointwise feasible thickness bounds (based on underlying dev constraints)

# Underlying_dev <= +U (cannot be higher than +U)

Tmin_point = D + df["LevelConformance_mm"] - U_upper

Tmax_if_underlying_within_tol = D + df["LevelConformance_mm"] + U_lower

too_thin_impossible = df["Thickness_mm"] < Tmin_point

high_and_thin = (df["LevelConformance_mm"] >= A_upper) & (df["Thickness_mm"] < D)

low_and_thick = (df["LevelConformance_mm"] <= -A_lower) & (df["Thickness_mm"] > D)

thick_gt_max_if_underlying_within_tol = df["Thickness_mm"] > Tmax_if_underlying_within_tol

# Flags

def _flag_row(i) -> str:

f = []

if pd.notna(df["Thickness_mm"].iloc[i]) and pd.notna(df["LevelConformance_mm"].iloc[i]):

if too_thin_impossible.iloc[i]:

f.append("Thickness<MinFeasible(D+LC-U)")

if high_and_thin.iloc[i]:

f.append("High&Thin")

if low_and_thick.iloc[i]:

f.append("Low&Thick_UnderlyingLowPossible")

if thick_gt_max_if_underlying_within_tol.iloc[i]:

f.append("Thick>MaxIfUnderlyingWithinTol(D+LC+U)")

return ";".join(f)

df["Flags"] = [_flag_row(i) for i in range(len(df))]

# Assessment (3-way)

df["Assessment"] = "GENUINE NONCONFORMANCE – physically possible"

df.loc[low_and_thick, "Assessment"] = "MAY BE ACCEPTABLE – underlying low NCR (thickness made up in asphalt)"

df.loc[too_thin_impossible | high_and_thin, "Assessment"] = "CHECK SURVEY ERROR – physically impossible"

# Deliverable table: only level-nonconforming rows

out = df[df["Is_Level_Nonconforming"]].copy()

# Add helpful audit columns

out["MinFeasibleThickness_mm (D+LC-U)"] = Tmin_point[df["Is_Level_Nonconforming"]].values

out["MaxIfUnderlyingWithinTol_mm (D+LC+U)"] = Tmax_if_underlying_within_tol[df["Is_Level_Nonconforming"]].values

out["DesignThickness_mm"] = D

out["AsphaltTolUpper_mm"] = A_upper

out["AsphaltTolLower_mm"] = A_lower

out["UnderlyingTolUpper_mm"] = U_upper

out["UnderlyingTolLower_mm"] = U_lower

return out, level_note

def df_to_csv_bytes(df: pd.DataFrame) -> bytes:

return df.to_csv(index=False).encode("utf-8-sig")

def df_to_csv_bytes_with_title(df: pd.DataFrame, title: Optional[str] = None) -> bytes:

"""Return CSV bytes with an optional title line at the top.

Title will be placed as the first line, followed by a blank line, then the CSV data.

"""

csv_body = df.to_csv(index=False)

if title:

title_line = f"Survey Report: {title}\n\n"

return (title_line + csv_body).encode("utf-8-sig")

return csv_body.encode("utf-8-sig")

# ---------------------------

# Sidebar inputs (MANDATORY FIRST STEP)

# ---------------------------

with st.sidebar:

st.header("Inputs (must confirm)")

D = st.number_input("Design asphalt thickness D (mm)", min_value=0.0, value=130.0, step=1.0)

st.markdown("**Asphalt level tolerance (mm)** — specify separate upper (high) and lower (low) bounds")

A_upper = st.number_input("Asphalt tolerance upper (+A, mm)", min_value=0.0, value=10.0, step=1.0)

A_lower = st.number_input("Asphalt tolerance lower (-A, mm)", min_value=0.0, value=10.0, step=1.0)

st.markdown("**Underlying layer tolerance (mm)** — specify separate upper (high) and lower (low) bounds")

U_upper = st.number_input("Underlying tolerance upper (+U, mm)", min_value=0.0, value=10.0, step=1.0)

U_lower = st.number_input("Underlying tolerance lower (-U, mm)", min_value=0.0, value=10.0, step=1.0)

st.markdown("---")

sign_ok = st.checkbox("Positive = high; Negative = low", value=True)

st.markdown("---")

upload = st.file_uploader("Upload CSV / Excel / PDF", type=["csv", "xlsx", "xls", "txt", "pdf"])

st.markdown("---")

st.subheader("PDF settings (digital tables)")

pdf_flavor = st.selectbox("Camelot flavor", ["stream", "lattice"], index=0)

pdf_pages = st.text_input("Pages", value="all", help='Examples: "all", "1", "1-3,5"')

st.markdown("**Header row finder**")

use_header_finder = st.checkbox("Auto-detect header row in each PDF table", value=True)

header_scan_rows = st.slider("Scan first N rows for header", min_value=5, max_value=60, value=25, step=5)

if not upload:

st.info("Upload a CSV/Excel/PDF containing Chainage, Offset, Level Conformance, and Thickness.")

st.stop()

if not sign_ok:

st.warning("This app assumes Positive=high and Negative=low. Tick the checkbox to proceed.")

st.stop()

# ---------------------------

# Load data

# ---------------------------

try:

name = upload.name.lower()

if name.endswith(".pdf"):

raw = extract_tables_from_pdf(

pdf_bytes=upload.getvalue(),

pages=pdf_pages,

flavor=pdf_flavor,

header_finder=use_header_finder,

header_scan_rows=header_scan_rows,

)

st.success("PDF tables extracted. Header row finder applied (if enabled).")

else:

raw = load_csv_excel(upload)

except Exception as e:

st.error(f"Could not read file: {e}")

st.stop()

raw = normalize_cols(raw)

st.subheader("Raw data preview (first 40 rows)")

st.dataframe(raw.head(40), use_container_width=True, height=360)

# ---------------------------

# Column mapping UI

# ---------------------------

st.subheader("1) Map columns")

chainage_guess = find_col(raw, ["Chainage", "Ch", "Chainage (m)", "Chainage_m"])

offset_guess = find_col(raw, ["Offset", "Off", "Offset (m)", "Offset_m"])

level_guess = find_col(raw, ["Level conformance", "LevelConformance", "Level conformance (mm)", "Conformance", "Level diff", "Delta", "Δ"])

thickness_guess = find_col(raw, ["Thickness", "Thickness (mm)", "AC Thickness", "Asphalt Thickness"])

col1, col2, col3 = st.columns(3)

with col1:

chainage_col = st.selectbox(

"Chainage column",

options=["(none)"] + list(raw.columns),

index=(1 + list(raw.columns).index(chainage_guess)) if chainage_guess in raw.columns else 0,

)

with col2:

offset_col = st.selectbox(

"Offset column",

options=["(none)"] + list(raw.columns),

index=(1 + list(raw.columns).index(offset_guess)) if offset_guess in raw.columns else 0,

)

with col3:

level_col = st.selectbox(

"Level conformance column (mm)",

options=["(none)"] + list(raw.columns),

index=(1 + list(raw.columns).index(level_guess)) if level_guess in raw.columns else 0,

)

st.markdown("**Thickness column (mm)** — required: provide the survey Thickness column in millimetres.")

thickness_col = st.selectbox(

"Thickness column (mm)",

options=["(none)"] + list(raw.columns),

index=(1 + list(raw.columns).index(thickness_guess)) if thickness_guess in raw.columns else 0,

)

if level_col == "(none)":

st.error("You must map a Level conformance column (mm).")

st.stop()

chainage_col = None if chainage_col == "(none)" else chainage_col

offset_col = None if offset_col == "(none)" else offset_col

level_col = level_col

thickness_col = None if thickness_col == "(none)" else thickness_col

top_rl_col = None

bot_rl_col = None

# ---------------------------

# Compute + classify

# ---------------------------

try:

df = raw.copy()

if not chainage_col:

df["Chainage"] = np.nan

chainage_col = "Chainage"

if not offset_col:

df["Offset"] = np.nan

offset_col = "Offset"

df["Chainage"] = to_numeric(df[chainage_col])

df["Offset"] = to_numeric(df[offset_col])

df, thickness_note = compute_thickness(df, thickness_col, top_rl_col, bot_rl_col)

tagged, level_note = classify_points_updated(

df,

D=float(D),

A_upper=float(A_upper),

A_lower=float(A_lower),

U_upper=float(U_upper),

U_lower=float(U_lower),

level_col=level_col,

)

except Exception as e:

st.error(f"Failed to process: {e}")

st.stop()

# ---------------------------

# Outputs

# ---------------------------

st.subheader("2) Assumptions (explicit)")

st.write(

{

"Design thickness D (mm)": float(D),

"Asphalt tolerance upper/lower (mm)": f"+{float(A_upper)}/-{float(A_lower)}",

"Underlying tolerance upper/lower (mm)": f"+{float(U_upper)}/-{float(U_lower)}",

"Sign convention": "Positive = high; Negative = low",

"Hard minimum feasible thickness (per-point)": "Tmin_point = D + LevelConformance - U (underlying cannot be higher than +U)",

"LOW & THICK handling": "Flagged as MAY BE ACCEPTABLE (subject to underlying low NCR)",

"Thickness source": thickness_note,

"PDF mode": f"Digital tables via Camelot ({pdf_flavor})",

"LevelConformance source": level_note,

"Header row finder": "Enabled" if use_header_finder else "Disabled",

}

)

total_nonconf = len(tagged)

n_check = int((tagged["Assessment"] == "CHECK SURVEY ERROR – physically impossible").sum())

n_genuine = int((tagged["Assessment"] == "GENUINE NONCONFORMANCE – physically possible").sum())

n_maybe = int((tagged["Assessment"] == "MAY BE ACCEPTABLE – underlying low NCR (thickness made up in asphalt)").sum())

st.subheader("3) Summary")

c1, c2, c3, c4 = st.columns(4)

c1.metric("Total level-nonconforming points", total_nonconf)

c2.metric("CHECK SURVEY ERROR", n_check)

c3.metric("GENUINE NONCONFORMANCE", n_genuine)

c4.metric("MAY BE ACCEPTABLE", n_maybe)

st.subheader("4) Tagged table (level-nonconforming points only)")

mandatory_cols = [

"Chainage",

"Offset",

"LevelConformance_mm",

"Thickness_mm",

"Thickness_Diff_vs_Design_mm",

"MinFeasibleThickness_mm (D+LC-U)",

"MaxIfUnderlyingWithinTol_mm (D+LC+U)",

"Assessment",

"Flags",

]

for col in mandatory_cols:

if col not in tagged.columns:

tagged[col] = np.nan

st.dataframe(tagged[mandatory_cols], use_container_width=True, height=560)

st.subheader("5) Download")

st.download_button(

label="Download tagged CSV",

data=df_to_csv_bytes_with_title(

tagged[mandatory_cols], title=os.path.splitext(upload.name)[0]

),

file_name=f"{re.sub(r'[^A-Za-z0-9_-]', '_', os.path.splitext(upload.name)[0])}_Asphalt_NCR_Checks_Tagged_Level_Nonconforming_Points.csv",

mime="text/csv",

)

st.caption(

"Interpretation: "

"CHECK SURVEY ERROR = physically impossible given underlying cannot be above +U. "

"MAY BE ACCEPTABLE = low & thick, consistent with underlying low (potentially accepted NCR)."

)